# Purpose of this Document

This document is a consolidation of the information contained within the mammoth TM MWS Thread here: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/

The purpose is to save newcomers (or people looking back for older information) hours of sifting through pages and pages of posts to get to what they are looking for or think they remember seeing ages ago and now realise is relevant to them.

Please keep edits or additions brief, to the point and consistent with the formatting.

So that information doesn’t get missed, please don’t just add in information from the current page, but start from the page mentioned below.

This was last updated when the thread was at Page 79.

# The Basics

Stock performance: ~310fps on green gas/propane.

Stock Outer barrel: 14.5” counter-clockwise threaded.

Recommended ammo weight: 0.28g

# Modifications/Upgrades

External

## Rails

Unless using a Dytak outer barrel, fitting a third-party rail system requires a PTS or ORGA barrel adapter ring. It buffers the castle nut providing compatibility with most third-party rails such as PTS etc.

G&P make a lot of accessories “For MWS” which should fit without an adapter (unconfirmed).

Buying Sources:

PTS adapter ring: https://www.fire-support.co.uk/product/pts-adapter-ring-for-marui-m4-mws-gbb

Internal

The most popular upgrades in terms of bang for buck or ration of effort to performance increase is the nub, inner barrel and to a lesser extent the bucking.

An excellent post on the relationship between the barrel, nub, arm and bucking here: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=354169

## Barrel

Stock inner barrel length: 250mm

Max inner barrel length (without a longer outer or suppressor): 370mm

With a 16” outer barrel, you can go 400-450mm inner.

One user suggested wrapping the inner barrel with PRFE tape for a snug fit, though it doesn’t seem like wobble is an issue that needs solving: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=315685

For a DMR build, users tend to opt for either the ORGA LEX 410mm 6.05 tightbore, or the Prommy 370mm 6.05mm.

At least one user has posted claiming that a 6.05mm offers superior accuracy over the 6.03mm with the MWS, though this seems largely anecdotal.

The Prommy barrel is a lot easier to get hold off that’s the Orga, especially if you prefer UK suppliers.

Either should boost FPS by around 20-30fps.

Buying Sources:

Prommy 370mm

https://www.fire-support.co.uk/product/prometheus-603mm-eg-inner-barrel-for-m4a1-mws-370mm

## Nub

The stock MWS nub is a hollow plastic one, meaning it’s ability to apply pressure is limited (due to the flex). A hugely popular upgrade that most users will agree is the single “best” upgrade when it comes to cost, effort and performance gain is the SuperNub.

### SiXG Super Nub

The SiXG Super Nub is made from solid brass and therefore has no flex, providing a more consistent application of pressure to the bucking compared to the stock hollow TM nub. The stock nub can successfully hop up to a maximum of 0.28g bb, whereas with the Super Nub the otherwise stock gun can hop beyond 0.4g bb, with most users using 0.43g or 0.46g.

Using 0.43g ammo, the MWS has is generally considered to be effective up to around 60m, 80m+ if combined with one of the barrels above.

Buying Sources:

https://sixgunsmithing.wordpress.com/2018/03/11/sixg-super-nub-for-marui-gbbr/

### Alternatives to the Super Nub:

- Omega nub (See post: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=316957)

- Element H nub (see below re horizontal inconsistency)

- TNT﻿ TR hop (see below re barrel compatibility)

- TNT APS-X, though the shape is noticeably flatter.

- Laylax Strike Hop Arm

#### Layla’s Strike Hop Arm

Eagle6 have manufactured a strike arm with integrated nub that is comparable to the SixG Super nub, though can’t find direct performance comparisons. Nobody seems to dispute this is a viable alternative though.

Buying Sources:

https://eagle6.co.uk/shop/catalog/product/view/id/4773/s/laylax-f-factory-m4a1-mws-strike-hop-arm-for-m4-gas-machine-gun-series/

### Prometheus Bridge Nub

In a DMR build, some users have opted for the Prometheus﻿ bridge nub. This seems to be longer and therefore offer a bigger contact patch, similar to r-hopping an AEG.

The problem users are encountering is that the longer nub won’t fit without modification to either the bucking window or the nub itself. It also doesn’t seem to be all that necessary even in a DMR build. https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=315685

### TNT & Element H

At least one user has opted for the TNT 50degree bucking. Whilst he initially paired this with the Element H nub he eventually replaced it with the longer Prometheus open. This required filing down to fit: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=353622

The same user later abandoned this route as he encounter horizontal inconsistency with shots.

Other users have commented that the TNT bucking needs to be paired with the Crazy Jet barrels: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=353958

## Bucking

Bucking comparison with stock versus some similar products: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=303973

Great post re concave versus flat bucking and how it interacts with the nub: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=351382

### Modify Tan Bucking

Modify tan bucking seems to be the outright favourite bucking, although some posts would suggest this is more for CQB or carbine length rifles rather than DMR: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=315651

One users’ comment on the Modify﻿﻿ Tan﻿﻿ fitted with Omega Nub: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=316957

Buying Sources:

https﻿://www.skirmshop.nl/en/modify-accurate-hop-u﻿p-bucking-1st-class-aircraft.html

https://www.longbowbb.co.uk/product-page/modify-gbb-accurate-hop-up-bucking

### TM Standard

Rated as excellent, combined with the SixG Super nub, seems to be comparable to the Modify Tan, reportedly with slightly less efficient air seal. However, in a more recent post DaveGolf (the dude behind the Super Nub) suggested the standard bucking may be preferable compared to silicone based bucking due to durability issues with other bucking not specifically designed for GBBRs. The post was a bit ambiguous though so this may not be the case: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=372511

### Maple Leaf

Various users have tried different ML buckings, including the Decepticon and the Super. Wolf’s DMR build was initially equipped with an ML Decepticon 60. The locking tab seemed to be causing problems however, so he upgraded to the “Super” which was the newer version, with no locking tab (just a guided runner impression).

### Autobot

AutoBot has been mentioned a few times, and was recommended along with the Omega Nub by Camo Raids.

Choose either 50 or 60 degrees and if going with the Decepticon and remove the locking tab. Seems to work best with the Omega nub: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=316876i8

### TNT

Wolf is running the TNT T-Hop in his DMR, recommends the new TR-Hop as it is new and improved apparently. But also made this comment regarding the larger contact patch needing a longer nub: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=351416

## Recommended/Common Upgrades:

### General Mods

Comment from DaveGolf on the worth while upfront upgrades: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=303934

### Green Gas Mod

The “green gas mod” involves a relatively easy mod to the magazine, cutting down a tube that surrounds the fill valve, increasing gas capacity. However, the stock magazine will empty around 2 mags on a single fill anyway so unless you’re determined to carry a speedloader in the field, there doesn’t seem to be any logical reason to do this.

Good information contained with this post: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=334660

Preferred method of a few users: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=374080

### Recoil Spring

Recommended upgrade is a stiffer recoil spring, but again this isn’t across the board recommended, like say, the Super Nub. Very much optional: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=354526

### Semi-Lock

The gun can be locked to semi by removing the contact patch on the fire selector via a simple exercise with a Dremmel, but the trigger mechanism is a bit of a hassle to remove from the lower receiver due to how tightly it’s packed: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=320961

## Recommended Upgrades for a DMR Build

One post on achieving a 450fps DMR build: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=372862

Another, following the above upgrades: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=372862

## Running the Gun

### Magazines

Available from Dave’s Custom Airsoft but a lot of users seem to report issues, such as inconsistent FPS (including reducing peak power), difficult seating the magazine in the magwell and otherwise. General consensus seems to be not to bother.

### Speedloaders

One user recommends the Hurricane﻿ Storm Load﻿er or MWS War﻿mag from softair ﻿websit﻿e but this has been generally lambasted as expensive crap by a lot of other users, citing the handle breaking off upon first usage etc.

Another user (who didn’t get on with the Hurricane) suggested the TM﻿ XL 470 speed﻿loader saying it’s cheap and works fine with the gas mag flip-down adapter that comes attached to it: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=372528

### Silent Fill (can help if propane is troublesome)

A few users have experienced issues using propane, it appears TM might have made some changes to the intake valve, but this isn’t confirmed.

The most consistent fix seems to be to fit o-rings onto the intake valve, making them silent fill, although this can mean you get less gas into the mag (as the hissing sound without the o-rings is the propane or green gas displacing the escaping air).

Buying Sources:

https://eagle6.co.uk/shop/catalog/product/view/id/3927/s/eagle6-airsoft-gas-magazine-entry-valve-o-rings-for-all-gas-guns/

## FPS Moderation

Best bet is an NPAS (allows the user to control the amount of gas that goes to propelling the BB, therefore controlling FPS).

There’s one silver WE version that also fits the MWS and a black MWS version. They both fit in the MWS but the black one holds it’s adjustment better.

The NPAS may rotate when the user is trying to adjust it, but Wolf has offered a suggestion on how to stop this, though it isn’t clear exactly what is involved: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=331566

Otherwise, rocket valves are available from (but not only) G&P. https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=369906

### Angry Gun MPA Nozzle

Angry gun launched an MPA nozzle in summer ‘19. Basically a nozzle with an integrated NPAS. Nobody has reported results from this yet.

Buying Sources:

https://www.mlemart.com/ra-npas-kit-for-marui-system/

## Gas

Some users use propane unless <0 degrees, then MAP gas (or Guarder power-up is comparable to MAP without the smell).

I personally run Abbey Vertex Predator gas because I have experienced a lot of issues with getting propane into the mags, plus propane seems to actually be more expensive now compared to the Abbey Gas. The Vertex gas has minimal lubricant mixed in and the lubricant isn’t silicone meaning is remains suspended in the gas (good thing) rather than sinking to the bottom of the mag, potentially then drawing out the lubricant from the o-rings at the top.

One retailer has said the MWS platform is fine with Black Gas/Guarder Power-Up Gas all year round, the only issue being keeping FPS under limit (not such as issue if going DMR of course).

## Maintenance:

Wolfs comprehensive maintenance guide: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=304756

Further discussion on types of maintenance products: https://airsoft-forums.uk/topic/40967-the-tm-mws-thread/?do=findComment&comment=344276

Eagle6 have a playlist on YouTube showing the complete disassembly and reassembly of the gun: https://www.youtube.com/playlist?list=PL3j0AoaNvjquQbxku0bnfJdoXd8_mby0U